#106 Dovetail Corrugated Anchor Specification

SECTION 04 00 00 MASONRY

Section 04 05 19.16 Masonry Anchors

Section 04 05 23 Masonry Accessories

PART 1: GENERAL

1.1 RELATED SECTIONS

A. Provide shop drawings for all product locations.

1.2 SUBMITTALS

A. Manufacturer Certificate of Compliance for materials.

B. Product Data: Manufacturer’s data sheet on each type of product 					    furnished.

PART 2: MANUFACTURER

2.1 MANUFACTURER

Heckmann Building Products

110 Richards Ave.

Norwalk, CT 06854-1685

800-621-4140

Email: info@heckmannanchors.com

Website:  www.heckmannanchors.com

2.2 APPLICATIONS

- Provide anchoring systems that comply with the Building Code Requirements for Masonry Structures TMS 402-22.

B. Stainless Steel AISI [Type 304] [or] [Type 316]

C. ASTM A240/A240M-15b Standard Specification for Chromium and Chromium-	Nickel Stainless Steel Plate, Sheet, and Strip for Pressure Vessels and for 	General Application.

D. ASTM A666-15 Standard Specification for Annealed or Cold-Worked 	Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar.

2.3 MATERIALS

Select gauge, length and steel type

106 DOVETAIL CORRUGATED ANCHOR [gauge] x 1 inch (25 mm) wide 	x [length from face of concrete]

[Type 304 Stainless Steel] or [Type 316 Stainless Steel]

PART 3: EXECUTION

A. Install as specified in applicable masonry section(s).

B. Anchors: Insert anchor at desired height into Dovetail Anchor Slot. Turn 90 degrees to lock into the slot. Place mortar above and below anchor in the mortar joint. [Specify spacing of anchors].